Assembly procedure of the Smart Clamp linear motor

Safety precautions

Tools

The following tools and supplies are required for performing the assembly procedure:

- Hex key (size 3)

- Hex key (size 4)

- Hex key (size 5)

- Fixture

Parts

The following parts are required for performing the assembly procedure:

- 1x 	Slider screw M8x25 (DIN 7984)

- 2x	Spherical washer M8 (DIN 6319c)

- 2x	Shaft screw M6x12 (ISO 4762)

- 1x	Front plate (0150-5005)

- 2x	Conical seat M8 (DIN 6319d)

- 1x	Dished washer 16/8,2/0,9 (DIN2093A)

- 1x	Slider (0150-1506)

- 2x	Hardened steel shafts for ball bearings (0150-5010)

- 3x	Clamping screw M5x18 (ISO 4762)

- 3x	Clamping cylinder (0150-5053)

- 1x	Guide block with ball bearings (0150-5002)

- 1x	Stator (0150-1252)

- 2x 	Washer M6 20/1,5

- 2x	Shaft screw M6x12 (ISO 7380)

- 1x	Brake hole cap (0160-0727)

- 1x 	Cooling fan 80x80mm

- 4x 	Square nut M4 (DIN 562)

- 4x 	Hexagonal spacer M4x16

- 4x	Screw M4x30 (ISO 4762)

- 1x 	Protective grid

Start the assembly procedure

- Make sure the assembly kit is available on the work table.

- Grab the black guide block from the table and position it in the fixture with the T-slots upwards and the 3 holes towards you.

- Grab the stator from the kit.

- Slide the stator carefully in the housing from the right side.

- Fix the stator in the housing:

  - Grab three clamping cylinders from the container on the table

  - Grab three clamping screws M5x18

  - Insert the clamping cylinders completely and put a screw in each of them.

  - Use the hex key (size 4) to fasten the screws.

- Grab the brake hole cap from the container and push it in the hole on the guide block.

- Assemble the slider:

  - Grab the tube with the slider from the kit.

  - Open the tube and carefully extract the slider rod. It is highly magnetic, so grasp it firmly.

  - Insert the slider into the stator from the left side. You’ll notice some resistance resulting from the magnetic effects.

  - Put the tube back in the kit.

- Insert the hardened steel shafts in the guide block from the right side.

- Put the front plate against the two shafts.

  - Make sure ‘This side up’ is visible after mounting. The text should be oriented towards the housing.

  - Fix the front plate to the shafts by using the two shaft screws M6x12 and the hex key (size 5).

- Prepare the slider screw:

  - Grab the slider screw M8x25 from the container.

  - Grab 2 spherical washers, 1 conical seat and 1 dished washer from the container.

  - Place the spherical washer on the slider screw and insert it into the front plate. The conical seat is milled into the front plate.

- Attach the front plate to the slider:

  - Place the conical seat, the spherical washer and the dished washer on the other side of the front plate onto the slider screw.

  - Screw the bolt into the slider rod end. Use a hex key (size 5) for this.

Assemble the fan for additional cooling

- Grab 4 square nuts M4 and 4 hexagonal spacers M4x16 from the containers on the table.

- Assemble spacers:

  - Slide the 4 square nuts into the T-slots on the guide block in the correct positions.

  - Screw the 4 hexagonal spacers onto the square nuts.

- Put the cooling fan on top of the spacers and adjust the spacer position if necessary.

- Put the grid on top of the cooling fan.

- Assemble cooling fan

  - Grab 4 screws M4x30 from the container on the table

  - Fix the 4 screws using the hex key (size 5)

- The assembly is finished.